L298N Motor Driver to Arduino Mega

| L298N Pin | Connect To |
| --- | --- |
| OUT1 | Red wire of the DC motor |
| OUT2 | White wire of the DC motor |
| ENA | Pin 3 on Arduino Mega (PWM control) |
| IN1 | Pin 4 on Arduino Mega |
| IN2 | Pin 5 on Arduino Mega |
| GND | GND (to Arduino GND and battery GND) |
| 12V | External power (6V–12V battery) |
| 5V | ❌ Do NOT use (especially not for servo!) |

DC Motor Encoder

| Encoder Wire | Connect To |
| --- | --- |
| Green (A signal) | Arduino Mega Pin 18 (Interrupt) |
| Yellow (B signal) | Arduino Mega Pin 19 (Interrupt) |
| Blue (VCC) | Arduino 5V |
| Black (GND) | Arduino GND |

Servo Motor to Arduino Mega

| Servo Wire | Connect To |
| --- | --- |
| Signal (Orange) | Pin 9 on Arduino Mega |
| VCC (Red) | ⚠️ External 5V–6V power source (not Arduino) |
| GND (Brown) | Common GND (Arduino GND + battery GND) |

Important: Don’t power the servo directly from the Arduino Mega — it doesn’t provide enough current. Use a battery pack or buck converter with shared ground.

Power Connections

- DC Motor → Powered by external source (6V–12V) through L298N

- Servo Motor → Separate regulated 5V–6V source (e.g., 2S LiPo with voltage regulator)

- Arduino Mega → Powered via USB or VIN (7V–12V regulated input)

Pins Summary (Arduino Mega)

| Purpose | Pin |
| --- | --- |
| DC Motor ENA (PWM) | Pin 3 |
| DC Motor IN1 | Pin 4 |
| DC Motor IN2 | Pin 5 |
| Servo Signal | Pin 9 |
| Encoder A | Pin 18 (optional) |
| Encoder B | Pin 19 (optional) |