DC Motor via L298N Module

(Connected to pins 3, 4, and 5)

L298N to Arduino Mega:

- IN1 → Pin 4 (controls motor direction)

- IN2 → Pin 5 (controls motor direction)

- ENA → Pin 3 (PWM pin, controls motor speed)

L298N Power:

- OUT1 & OUT2 → Connect to DC motor wires

- 12V / VCC → Connect to external power supply (6V–12V battery)

- GND → Connect to Arduino GND

- 5V (if jumper is present) → Optional; powers logic (you can use it if your battery is strong enough)

Servo Motor (Steering)

(Connected to pin 9)

Servo Motor to Arduino Mega:

- Signal (usually orange or yellow) → Pin 9

- VCC (red) → External 5V–6V power source

- GND (brown/black) → Connect to common GND with Arduino

Servo motors need a separate power supply if they draw significant current. Always connect the grounds together (Arduino GND and power source GND).